# ANNEX II + III:	 TECHNICAL SPECIFICATIONS + TECHNICAL OFFER

- Contract title: SUPPLY OF MOBILE LABORTAORIES; MOBILE VETERINARY LABORATORY VEHICLE (LOT 1), AND EMBRYO TRANSFER FACILITY VEHICLE (LOT 2).

Publication reference: MOBIP/SUP/005

Columns 1-2 should be completed by the Contracting Authority

Columns 3-4 should be completed by the tenderer

Column 5 is reserved for the evaluation committee

Annex III - the Contractor's technical offer

The tenderers are requested to complete the template on the next pages:

- Column 2 is completed by the contracting authority shows the required specifications (not to be modified by the tenderer),

- Column 3 is to be filled in by the tenderer and must detail what is offered (for example the words ‘compliant’ or ‘yes’ are not sufficient)

- Column 4 allows the tenderer to make comments on its proposed supply and to make eventual references to the documentation

The eventual documentation supplied should clearly indicate (highlight, mark) the models offered and the options included, if any, so that the evaluators can see the exact configuration. Offers that do not permit to identify precisely the models and the specifications may be rejected by the evaluation committee.

The offer must be clear enough to allow the evaluators to make an easy comparison between the requested specifications and the offered specifications.

LOT 1: Mobile Veterinary Laboratory Vehicle

| 1. Item number and Description of the Vehicle major components | 2. Specifications required | 3. Specifications offered | 4. Notes, remarks, ref to documentation | 5. Evaluation committee’s notes |
| --- | --- | --- | --- | --- |
| - GENERAL REQUIREMENTS | Specialised and mobile veterinary, food safety and testing lab of latest, high range and performing, Diesel engine series, Four Cylinders, water - Cooled and equipped with all accessories and equipment for both field and specific operations and equipped with Four Wheel Drive mechanism |  |  |  |
| - Year of Manufacture | 2019 or latest model series |  |  |  |
| Country Of Origin /Manufacture | With reputable network and inventory in Uganda inclusive of the lab equipment and accessories |  |  |  |
| Model or Brand type | Standard and Latest Model series |  |  |  |
| ENGINE SPECIFICATIONS AND PARAMETERS: |  |  |  |  |
| Engine Type | Diesel, Common Rail fuel injection system, Direct Injection |  |  |  |
| Engine Capacity, cc | Displacement: 2450c.c. |  |  |  |
| Fuel System and accessories | Common Rail and Direct Injection |  |  |  |
| Fuel Type | Diesel |  |  |  |
| Fuel Tank Capacity with lockable and secured cup, Min, Ltrs | 75 |  |  |  |
| No of Cylinders and arrangement | Four and inline arranged Cylinders |  |  |  |
| Engine Power Output, min.Kw @rated Rpm | 125Kw@rated rpm |  |  |  |
| Air Intake | Replaceable air Cleaner and Cyclone Type |  |  |  |
| Cooling System | Closed and forced water system with heavy duty and tropicalised radiator, reservoir and protected |  |  |  |
|  | High Altitude Compensator |  |  |  |
| - Drive Train/Systems |  |  |  |  |
| Body Type and Description | - Super long and High Roof vehicle with provision of cage, panel or roof carrier, equipped with clean energy panels and all required accessories. The panel should be foldable coupled with in-built solar or other clean energy mechanism as alternative energy source to power the specialized equipment, lab, clinic, instruments and equipment. Should have separate surgery unit or suite. - Cabin layout of the mobile specialized and expert laboratory vehicle shall correspond to all other requirements and not limited to: - The surgery unit or suite space in the vehicle should have be separate from the cabin. - The surgery suite is isolated and accessed only from outside through the backdoor or side door to minimize any contamination risk and to provide space or storage for equipment, materials, and samples as well as performing of all the veterinary clinics operations and activities. - Floor should be made of plywood or non slip hospital grade flooring with rubberized lancoin flooring covers the full floor for heavy duty rear - Surfaces must allow easy cleaning and decontamination. - Rough terrain transportation of equipment and sample capabilities (e.g. special transportation boxes for all analytical instruments, fastening and fixation of cargo in the storage area) - The entire built vehicle body should me all unique requirements for all veterinary practices - Accommodate all the wide variety of equipment options and upgraded features and standard veterinary features as detailed in the equipment and instruments requirements - Foldable rear tails doors open able at 180 Degrees - Interior Roof racks with LED ceiling lights provision |  |  |  |
| Transmission Type and Transfer Case | 4WD, Manual and Synchronized Gear box |  |  |  |
| Gear Arrangement, Shifting and Number of Gears, Min | Manual Floor Shift, 6F and 1R, Six Speed gear box with differential lock at the rear differential unit |  |  |  |
| - Steering System Type | Integral Type, Power steering, RHD with Strong and forked wheel with tilt adjuster mechanism |  |  |  |
| - Brakes System and actuation: |  |  |  |  |
| Front | Ventilated Disc, fluid actuated |  |  |  |
| Rear | Ventilated Disc, fluid actuated |  |  |  |
| Security | ABS Brakes |  |  |  |
| Parking Brakes | Independent and Mechanically operated with cable actuation |  |  |  |
| - Safety and Security |  |  |  |  |
|  | Front Dual SRS Air Bags – For Both Front rear Passengers and Driver with a knee cap included |  |  |  |
|  | Laminated Windscreen with sun shields provision |  |  |  |
|  | Heavy duty Front Bull Bar (Covering Entire width of vehicle compliant to Air Bags) with all lighting and safety provisions |  |  |  |
|  | Rear guard (Heavy Duty Type) and Tail Lights Protection |  |  |  |
| Body Protection | Side and rear doors access grips |  |  |  |
|  | Engine Immobilizer that only accepts engine start with authorized keys, Floor carpets |  |  |  |
| - Seating Capacity: |  |  |  |  |
| Seating Capacity, Min (Number) | 4 Seater all with head rests |  |  |  |
| Protection | Leather Seats with preferably leather bolsters and cushion covers only |  |  |  |
| Seatbelts with locks, covers with belt warning buzzer | 3 point locking, retractable and firmly secured |  |  |  |
| - Chassis and Body: Dimensions and Capacities | Heavy duty chassis that allows all other equipments and accessories to be fitted |  |  |  |
| Ground Clearance, minimum, mm | 240 |  |  |  |
| Wheel Base, Min.mm | 3100 |  |  |  |
| Tyre, Wheels and Rims |  |  |  |  |
| Tyres, wheels/Rims and accessories Sizes, Min | Rim size: R17 Full size with special lock nut on all heavy duty steel or alloy Wheels/Rims inclusive of spare tyre with similar dimensions |  |  |  |
| Vehicle Towing and support Capacity, Kg, Min | Braked Trailer: 1550 |  |  |  |
|  | Un Braked Trailer: 750 |  |  |  |
| - Suspension System | Heavy Duty Suspension for both front and rear |  |  |  |
| Front | Heavy duty and high performing suspension systems equipped with heavy duty telescopic Shock Absorbers with all related accessories/parameters |  |  |  |
| Rear | Heavy duty suspension with telescopic Shock Absorbers inclusive of all other accessories or features |  |  |  |
| - Electrical System | All 12Voltage System |  |  |  |
| Battery Size and Type, Volts/AH | 12V, Min 85AH Maintenance Free type for the main engine requirement and one of similar type and condition but of 100AH for other accessories application with stand by Gen.Set provision |  |  |  |
| Ignition | Wireless Door entry, Electronic and Lockable |  |  |  |
| Lights and accessories | 2 Head Lamps with halogen bulbs, Indicator lights, Two lights at the rear (with wire netting protection), and interior roof lights for both front and rear. Also provision for all other lighting points for lab, cabinet and other related requirements |  |  |  |
| - OTHER STANDARD EQUIPMENT: | ACCESSORIES WITH ATTACHMENTS (SAFETY, CONFORT AND UTILITY) |  |  |  |
|  | Air Conditioner, halogen head lamps |  |  |  |
|  | Engine Immobilizer |  |  |  |
|  | Tachometer, Revolution Counter, Digital Clock |  |  |  |
|  | Odometer, Speedometer |  |  |  |
|  | AM/FM Radio Cassette/CD player with blue tooth or latest technology |  |  |  |
|  | 2 Sun visors, 2 Outside rear side view mirrors, 1 inside rear view mirror with power tilt and retractable |  |  |  |
|  | 4 Units of Mudguards with flaps for both front and rear |  |  |  |
|  | Tinted laminated windshield, 2-speed wipers with washer fluid tank |  |  |  |
|  | Heater and defroster (fog and mist) |  |  |  |
|  | Power Windows with anti trapping device |  |  |  |
|  | Central Locking System with Keyless Entry |  |  |  |
| Upholstery | Cloth Upholstery, Leather seats with head rests |  |  |  |
|  | Lockable Glove Box compartment and High altitude compensator |  |  |  |
| Number of Doors | Four (Two front, side and tail gate with interior locking units) with standard locking units |  |  |  |
| N. Operations, Maintenance and Laboratory Equipment, Tools, Kits and Accessories | Standard maintenance tool kit(Spanners, Pliers, screw drivers, test lamp), fire extinguisher and Wheel jack (2kgs, Hydraulic assisted), fastener or wheel Spanner and wedge. First Aid box should be firmly fixed and fully equipped Towing Hook at both front and rear, Reflective Triangle, Emergency lights and protective or reflective aprons. CCTV, GPS and surveillance camera should be provided Specialised Surgery, clinic and Laboratory equipment and Upgraded features in the cab. includes: - CR 4000 - ULTRA 12040Hf Portable X – Ray unit - ULTRA 9030Hf Portable X-Ray unit - DR wizard 14 x 17 cassette size wireless Flat Panel - Dental scaler/polisher - Anesthesia machine with vaporiser - Active Gas Evacuation system - Surgery and working lights - Auto Clave - Blood and other subustence equipment - Pulse Oximeter - Digital Radiology - Swift microscopes - Roof Ac - Rear heat exchanger - Side attachnments - Rear Back Up Camera - Exterior/Interior Graphics - Exterior outlets Other Standard Clinic Features - Battery optimal Gel cell Deep Cycle - Ply wood Floor w/Non – Slip Hospital Grade Flooring - 7000w Gas Generator - 25ft 50AMP Power Cord - Superior Cabinetry & Drawers Custom – made for mobile use - Roof Mount A/C 120V/12V Norcold 8cc, refigerator with freezer - 40Ltrs Fresh water Tank and Grey Water Tank each - Hot Water heater - Pharmaceutical Lock Drawers - S/S Sing with Gooseneck Faucent - Water Pump and Filter w/ Heater - Electic Heater (Portable) - Automatic Transfer 120V Gen./Shore - Cages with Divider (2) - Flat Top Surgery Table 50’’ in Surgery Suite (V Top Optional) - Windows w/Blinds (03) - Entrance Grab Handle - Flip Down Exam / Prep Table - Manual Ceiling Vent - 30’’ Heavy Duty Entry Door with Dead Bolt & Window - Padded Bench Seat Near Entrace Sliding Door CABIN LAYOUT Specific requirements for the cabin arrangement are as follows: - Surfaces must allow easy cleaning and decontamination. - Rough terrain transportation of equipment and sample capabilities (e.g. special transportation boxes for all analytical instruments, fastening and fixation of cargo in the storage area). Voice and data communication systems should be incorporated with: - A modem capable of connecting to GSM 900/1800/1900 and UMTS - networks and supporting of GPRS/3G/HSDPA/EDGE data transfer standards |  |  |  |
| O. Warranty/Security: | 100,000kms (min) un limited performance warranty for the vehicle and its accessories with three years warranty for all the specialized and mobile lab equipment (Warranty and ISO Certificates must be provided) |  |  |  |
| P. After sales Service | Must provide well equipped maintenance workshop and facilities, maintenance parts stock availability for the last five years, Out lets and networked dealership in the country with professional and skilled staff |  |  |  |
| Q. Training Personnel | Must provide training programme and schedule for over 80hrs for 5People especially on, use, operation, maintenance, safety and cruise controls of the vehicles, equipment, devices and veterinary. This must include on road training for the first 25,000Kms facilities for the selected staff |  |  |  |
| R. Manuals and maintenance books in English | Original Operation, driving and maintenance brochures, manuals must be provided |  |  |  |
| S. Other Requirements | Comprehensive and costed Preventive maintenance programme for 150,000kms, spare parts and accessories availability, Maintenance facilities, skilled or professional personnel in the country (Must show evidence of locations and availability at least in 3 regions) for the last five years - Preventive maintenance Spare parts, accessories requirements and details for over 2yrs or 80,000Kms Also provide the costs and schedule for 2years or 50,000kms of routine maintenance for both the equipment and truck |  |  |  |

XXXXXX

LOT 2: Embryo Transfer Facility Vehicle

| 1. Item number and Description of the Vehicle major components | 2. Specifications required | 3. Specifications offered | 4. Notes, remarks, ref to documentation | 5. Evaluation committee’s notes |
| --- | --- | --- | --- | --- |
| - GENERAL REQUIREMENTS | Specialised and mobile veterinary, surgery or embryo clinic /Lab should be of latest, high range and performing models, Diesel engine, Four Cylinders, water - Cooled and equipped with all accessories and equipment for both field and specific operations and equipped with Four Wheel Drive mechanism |  |  |  |
| - Year of Manufacture | 2019 or latest model series |  |  |  |
| Country Of Origin /Manufacture | With reputable network and inventory in Uganda inclusive of the lab equipment and accessories |  |  |  |
| Model or Brand type | Standard and Latest Model series |  |  |  |
| ENGINE SPECIFICATIONS AND PARAMETERS: |  |  |  | ENGINE SPECIFICATIONS AND PARAMETERS: |
| Engine Type | Diesel, Common Rail fuel injection system, Direct Injection |  |  |  |
| Engine Capacity, Min cc | Displacement: 2450c.c. |  |  |  |
| Fuel System and accessories | Common Rail and Direct Injection |  |  |  |
| Fuel Type | Diesel |  |  |  |
| Fuel Tank Capacity with lockable and secured cup, Min, Ltrs | 75 |  |  |  |
| No of Cylinders and arrangement | Four and inline arranged Cylinders |  |  |  |
| Engine Power Output, min.Kw @rated Rpm | 125Kw@rated rpm |  |  |  |
| Air Intake | Replaceable air Cleaner and Cyclone Type |  |  |  |
| Cooling System | Closed and forced water system with heavy duty and tropicalised radiator, reservoir and protected |  |  |  |
|  | High Altitude Compensator |  |  |  |
| - Drive Train/Systems |  | - Drive Train/Systems |  |  |
| Body Type and Description | Super long and High Roof vehicle with provision of cage, panel or roof carrier, equipped with clean energy panels and all required accessories. The panel should be foldable coupled with in-built solar or other clean energy mechanism as alternative energy source to power the specialized equipment, lab, clinic, instruments and equipment. Should have separate surgery unit or suite. Cabin layout of the mobile specialized and expert laboratory vehicle shall correspond to all other requirements and not limited to: - The surgery unit or suite space in the vehicle should have be separate from the cabine. - The surgery suite is isolated and accessed only from outside through the backdoor or side door to minimize any contamination risk and to provide space or storage for equipment, materials, and samples as well as performing of all the veterinary clinics operations and activities. - Floor should be made of plywood or non slip hospital grade flooring with rubberized lancoin flooring covers the full floor for heavy duty rear - Surfaces must allow easy cleaning and decontamination. - Rough terrain transportation of equipment and sample capabilities (e.g. special transportation boxes for all analytical instruments, fastening and fixation of cargo in the storage area) - The entire built vehicle body should me all unique requirements for all veterinary practices - Accommodate all the wide variety of equipment options and upgraded features and standard veterinary features as detailed in the equipment and instruments requirements - Foldable rear tails doors open able at 180 Degrees - Interior Roof racks with LED ceiling lights provision |  |  |  |
| Transmission Type and Transfer Case | 4WD, Manual and Synchronized Gear box |  |  |  |
| Gear Arrangement, Shifting and Number of Gears, Min | Manual Floor Shift, 6F and 1R, Six Speed gear box with differential lock at the rear differential unit |  |  |  |
| - Steering System Type | Integral Type, Power steering, RHD with Strong and forked wheel with tilt adjuster mechanism |  |  |  |
| - Brakes System and actuation: |  | - Brakes System and actuation: |  |  |
| Front | Ventilated Disc, fluid actuated |  |  |  |
| Rear | Ventilated Disc, fluid actuated |  |  |  |
| Security | ABS Brakes |  |  |  |
| Parking Brakes | Independent and Mechanically operated with cable actuation |  |  |  |
| - Safety and Security |  |  |  |  |
|  | Front Dual SRS Air Bags – For Both Front rear Passengers and Driver with a knee cap included |  |  |  |
|  | Laminated Windscreen with sun shields provision |  |  |  |
|  | Heavy duty Front Bull Bar (Covering Entire width of vehicle compliant to Air Bags) with all lighting and safety provisions |  |  |  |
|  | Rear guard (Heavy Duty Type) and Tail Lights Protection |  |  |  |
| Body Protection | Side and rear doors access grips |  |  |  |
|  | Engine Immobilizer that only accepts engine start with authorized keys, Floor carpets |  |  |  |
| - Seating Capacity: |  |  |  |  |
| Seating Capacity, Min (Number) | 4 Seater all with head rests |  |  |  |
| Protection | Leather Seats with preferably leather bolsters and cushion covers only |  |  |  |
| Seatbelts with locks, covers with belt warning buzzer | 3 point locking, retractable and firmly secured |  |  |  |
| - Chassis and Body: Dimensions and Capacities | Heavy duty chassis that allows all other equipments and accessories to be fitted |  |  |  |
| Ground Clearance, minimum, mm | 240 |  |  |  |
| Wheel Base, Min.mm | 3100 |  |  |  |
| - Tyre, Wheels and Rims |  |  |  |  |
| Tyres, wheels/Rims and accessories Sizes, Min | Rim size: R17 Full size with special lock nut on all heavy duty steel or alloy Wheels/Rims inclusive of spare tyre with similar dimensions |  |  |  |
| Vehicle Towing and support Capacity, Kg, Min | Braked Trailer: 1550 |  |  |  |
|  | Un Braked Trailer: 750 |  |  |  |
| - Suspension System | Heavy Duty Suspension for both front and rear |  |  |  |
| Front | Heavy duty and high performing suspension systems equipped with heavy duty telescopic Shock Absorbers with all related accessories/parameters |  |  |  |
| Rear | Heavy duty suspension with telescopic Shock Absorbers inclusive of all other accessories or features |  |  |  |
| - Electrical System | All 12Voltage System |  |  |  |
| Battery Size and Type, Volts/AH | 12V, Min 85AH Maintenance Free type for the main engine requirement and one of similar type and condition but of 100AH for other accessories application with stand by Gen.Set provision |  |  |  |
| Ignition | Wireless Door entry, Electronic and Lockable |  |  |  |
| Lights and accessories | 2 Head Lamps with halogen bulbs, Indicator lights, Two lights at the rear (with wire netting protection), and interior roof lights for both front and rear. Also provision for all other lighting points for lab, cabinet and other related requirements |  |  |  |
| - OTHER STANDARD EQUIPMENT: | ACCESSORIES WITH ATTACHMENTS (SAFETY, CONFORT AND UTILITY) |  |  |  |
|  | Air Conditioner, halogen head lamps |  |  |  |
|  | Engine Immobilizer |  |  |  |
|  | Tachometer, Revolution Counter, Digital Clock |  |  |  |
|  | Odometer, Speedometer |  |  |  |
|  | AM/FM Radio Cassette/CD player with blue tooth or latest technology |  |  |  |
|  | 2 Sun visors, 2 Outside rear side view mirrors, 1 inside rear view mirror with power tilt and retractable |  |  |  |
|  | 4 Units of Mudguards with flaps for both front and rear |  |  |  |
|  | Tinted laminated windshield, 2-speed wipers with washer fluid tank |  |  |  |
|  | Heater and defroster (fog and mist) |  |  |  |
|  | Power Windows with anti trapping device |  |  |  |
|  | Central Locking System with Keyless Entry |  |  |  |
| Upholstery | Cloth Upholstery, Leather seats with head rests |  |  |  |
|  | Lockable Glove Box compartment and High altitude compensator |  |  |  |
| Number of Doors | Four (Two front, side and tail gate with interior locking units) with standard locking units |  |  |  |
| N. Operations, Maintenance and Laboratory Equipment, Tools, Kits and Accessories | Standard maintenance tool kit(Spanners, Pliers, screw drivers, test lamp), fire extinguisher and Wheel jack (2kgs, Hydraulic assisted), fastener or wheel Spanner and wedge. First Aid box should be firmly fixed and fully equipped Towing Hook at both front and rear, Reflective Triangle, Emergency lights and protective or reflective aprons. CCTV, GPS and surveillance camera should be provided Specialised Surgery, clinic and Laboratory equipment and Upgraded features in the cab. includes: - CR 4000 - ULTRA 12040Hf Portable X – Ray unit - ULTRA 9030Hf Portable X-Ray unit - DR wizard 14 x 17 cassette size wireless Flat Panel - Dental scaler/polisher - Anesthesia machine with vaporiser - Active Gas Evacuation system - Surgery and working lights - Auto Clave - Blood and other subustence equipment - Pulse Oximeter - Digital Radiology - Swift microscopes - Roof Ac - Rear heat exchanger - Side attachnments - Rear Back Up Camera - Exterior/Interior Graphics - Exterior outlets Other Standard Clinic Features - Battery optimal Gel cell Deep Cycle - Ply wood Floor w/Non – Slip Hospital Grade Flooring - 7000w Gas Generator - 25ft 50AMP Power Cord - Superior Cabinetry & Drawers Custom – made for mobile use - Roof Mount A/C 120V/12V Norcold 8cc, refigerator with freezer - 40Ltrs Fresh water Tank and Grey Water Tank each - Hot Water heater - Pharmaceutical Lock Drawers - S/S Sing with Gooseneck Faucent - Water Pump and Filter w/ Heater - Electic Heater (Portable) - Automatic Transfer 120V Gen./Shore - Cages with Divider (2) - Flat Top Surgery Table 50’’ in Surgery Suite (V Top Optional) - Windows w/Blinds (03) - Entrance Grab Handle - Flip Down Exam / Prep Table - Manual Ceiling Vent - 30’’ Heavy Duty Entry Door with Dead Bolt & Window - Padded Bench Seat Near Entrace Sliding Door CABIN LAYOUT Specific requirements for the cabin arrangement are as follows: - Surfaces must allow easy cleaning and decontamination. - Rough terrain transportation of equipment and sample capabilities (e.g. special transportation boxes for all analytical instruments, fastening and fixation of cargo in the storage area). Voice and data communication systems should be incorporated with: - A modem capable of connecting to GSM 900/1800/1900 and UMTS - networks and supporting of GPRS/3G/HSDPA/EDGE data transfer standards |  |  |  |
| O. Warranty/Security: | 100,000kms (min) un limited performance warranty for the vehicle and its accessories with three years warranty for all the specialized and mobile lab equipment (Warranty and ISO Certificates must be provided) |  |  |  |
| P. After sales Service | Must provide well equipped maintenance workshop and facilities, maintenance parts stock availability for the last five years, Out lets and networked dealership in the country with professional and skilled staff |  |  |  |
| Q. Training Personnel | Must provide training programme and schedule for over 80hrs for 5People especially on, use, operation, maintenance, safety and cruise controls of the vehicles, equipment, devices and veterinary. This must include on road training for the first 25,000Kms facilities for the selected staff |  |  |  |
| R. Manuals and maintenance books in English | Original Operation, driving and maintenance brochures, manuals must be provided |  |  |  |
| S. Other Requirements | Comprehensive and costed Preventive maintenance programme for 150,000kms, spare parts and accessories availability, Maintenance facilities, skilled or professional personnel in the country (Must show evidence of locations and availability at least in 3 regions) for the last five years - Preventive maintenance Spare parts, accessories requirements and details for over 2yrs or 80,000Kms Also provide the costs and schedule for 2years or 50,000kms of routine maintenance for both the equipment and truck |  |  |  |

The detailed technical evaluation will examine the technical specification of the items offered in column 3 and determine whether this meets the minimum specification in column 2.  Bidders must complete column 3 or the bid will be rejected.  Bidders are required to include the photographs/technical literature and original brochures to support the details provided in column 3 otherwise without the documents stated in the bid will be rejected.